Formative Quiz – Chapter 3

Student ID: __20210661____    Name: ___Seungjun Oh__   Date: _2026.03.09.__

1. (Concept)
What is process capacity?
Process capacity is strictly defined as the minimum capacity of all the resources in your system. It represents the absolute maximum output that your entire process can achieve.

2. (Concept)
What is a bottleneck in a process?
The bottleneck is the weakest link in your process—the specific step or piece of equipment that has the lowest capacity and ultimately limits the speed of the entire system.

3. (True / False)
A process can produce more than its bottleneck capacity. (False)

4. (Concept)
What is the difference between flow rate and capacity?
  Capacity is the maximum amount of work the process could theoretically handle.

  Flow rate is the actual amount of units actively moving through the system, which is dictated by the minimum of your available input, your demand, or your process capacity.

5. (Concept)
If demand is lower than capacity, is the process demand-constrained or supply-constrained?
It is demand-constrained. Your system has the physical capacity to do more, but the actual output (flow rate) is limited by the lower demand.

6. (Simple Calculation)
A machine can process 30 units per hour. What is its daily capacity if it operates for 8 hours?
240 units.

7. (Concept)
What does utilization measure?
Utilization measures how busy a resource actually is. It is calculated by dividing your actual flow rate by your capacity, and importantly, standard utilization can never exceed 100%.

8. (True / False)
The bottleneck is the resource with the highest utilization. (False)

9. (Concept)
What does implied utilization compare?
Implied utilization compares your pure demand against your actual capacity. It shows how much workload is hitting a resource regardless of whether the resource can actually handle it, which is why it is the only metric that can exceed 100%.

10. (Application)
If demand suddenly increases above capacity, what will most likely happen in the process?
You will start to accumulate a backlog. Since the bottleneck physically cannot process the work fast enough, tasks will pile up and create a queue or waitlist.